Welcome to the field! You’ve got the gear, and now it’s time to put it to use. This guide is your "pre-game" playbook to ensure your first solo run is a success. Think of this as a repeatable system—once you master the flow, you can handle any property with confidence.

### Lesson 1: Pre-Call Prep and Arrival (The First 5 Minutes)

Core Concept: Success begins before you even start the engine. Validating your order and preparing your tech prevents "dry runs" (trips where you can’t complete the job) and ensures you stay safe.

Step-by-Step Instructions:

- Verify the Order: Double-check the address, unit number, and gate codes; confirm the inspection type and due time.

- Confirm Rules: Check if a "call ahead" is required and verify if you are authorized to enter the property or contact neighbors.

- Tech Check: Charge your devices, clear storage, and log in to the portal while you still have a strong signal.

- Arrival Protocol: Park for a quick exit and perform a safety scan for hazards like loose dogs or suspicious activity.

Example from the Sources: In a typical sample job, the inspector confirms the template requirements (like the ABC number field) and shares their route with an emergency contact before arriving at the property.

Quick Win: Today, open your inspection app and confirm your login works. Download any offline templates you might need so you aren't stuck without data in the field.

Warning Signs: Watch out for "No Entry" orders where you might be tempted to knock—always stick to the entry authorization specified in your portal to avoid trespassing accusations.

### Lesson 2: Execute a Clean Occupancy Inspection

Core Concept: Your goal is to provide neutral, factual evidence of whether a property is occupied. You do this through a consistent "Four-Side Loop" that captures the entire exterior.

Step-by-Step Instructions:

- Identity Proof: Capture a tight shot of the house number followed by a wide shot showing the address in context with the front façade.

- The Four-Side Loop: Walk the property in a consistent flow (Front → Right → Rear → Left), capturing full wall lengths, rooflines, and outbuildings.

- Utility Check: Photograph all utility meters (electric, gas, water) to show if they are active or have been pulled.

- Look for Indicators: Document neutral signs like car presence, trash service activity, or accumulated mail/notices.

Example from the Sources: If a loose dog prevents you from completing the loop, document it from a distance. Use neutral notes like "Accumulated flyers at entry" or "No visible personal property through windows" rather than guessing the property is "abandoned."

Quick Win: Practice your "Four-Side Loop" at your own home today. See if you can capture all four sides and your utility meters in under five minutes.

Warning Signs: Avoid "Occupancy Guessing." Never mark a property as vacant without specific photo support, such as broken windows or an overgrown lawn.

### Lesson 3: Handle Interior Authorized Inspections

Core Concept: When entry is authorized, your mission is to document the condition of every room using a logical flow and accurate measurements. Consistency in your tools and units is key to preventing disputes.

Step-by-Step Instructions:

- Room Overview: Take at least one wide-angle overview shot of every room (living areas, bedrooms, kitchen).

- Maintain Measurement Integrity: Use the same tool (laser or tape) throughout the job and measure wall-to-wall at floor level.

- Document Defects: If you see damage, take a context shot first, then a close-up of the detail.

- Mechanicals: Capture the main electrical panel and other mechanical areas if required by the spec.

Example from the Sources: If you are measuring for a dispute, capture a photo that includes the tool display showing the measurement to make the data indisputable.

Quick Win: Pick one room in your house and measure it at floor level. Practice labeling the measurement (e.g., "North Wall - 12' 6''") so you don't forget which wall is which later.

Warning Signs: Never mix measurement units (like switching between feet/inches and decimal feet) or measure diagonals instead of straight walls.

### Lesson 4: Document Loss Draft and Repair Progress

Core Concept: Loss draft inspections focus on the progress of repairs after a claim. You must distinguish between materials sitting on-site and work that has actually been installed.

Step-by-Step Instructions:

- Quantify Damage: Document the dimensions of holes, missing drywall, or missing shingles.

- Identify Materials: Take photos of materials delivered to the site (like stacks of drywall or flooring).

- Verify Quality: Capture "work quality indicators" to show that repairs are being done professionally.

- Before/After: If possible, provide photos that show the current state of a previously documented defect.

Example from the Sources: When documenting repairs, take a "detail close-up with a scale reference" (like a ruler or a coin) to show the exact size of the repair or remaining damage.

Quick Win: Practice taking a "context then detail" photo set of a small repair or item in your home to ensure both the location and the detail are clear.

Warning Signs: Don't just take a close-up of a repair; without a wide context shot, the coordinator won't know which room the photo belongs to.

### Lesson 5: Upload Photos that Pass First Review

Core Concept: A "coordinator-proof" submission is organized, labeled, and complete. Your goal is to make it so the reviewer doesn't have to hunt for information or call you back.

Step-by-Step Instructions:

- Gallery Scan: Before leaving the site, scan your photos for blur, darkness, or missing required shots.

- Template Order: Upload photos in the exact order requested by the portal—do not make the coordinator search for the address shot.

- Caption Everything: Use standardized captions and ensure all mandatory fields (like the ABC number) are filled.

- Submission Proof: Always take a screenshot of the "Submission Successful" screen with the Job ID and timestamp.

Example from the Sources: In the sample job walkthrough, the inspector encountered an app failure. They screenshotted the error, retried on a hotspot, and saved the final confirmation screenshot as insurance.

Quick Win: Create a "Job Proof" folder on your phone or computer where you can store your confirmation screenshots for every job you complete.

Warning Signs: Leaving required fields blank is the number one way to get a rejection. If you can't provide data (e.g., a dog prevented access), explain why in the notes.

### Lesson 6: Respond to Rejections Like a Pro

Core Concept: Rejections are part of the learning curve. Professionalism means responding quickly, documenting why a photo might be missing, and providing a clear "Revision Version" note.

Step-by-Step Instructions:

- Identify the Gap: Review the coordinator's notes to see exactly what is missing (e.g., "missing rear photo" or "blurry address").

- Reference Your Proof: Check your confirmation screenshots or error logs if the rejection seems like a technical glitch.

- Provide context: If a photo is impossible to get (e.g., a "Beware of Dog" sign or a locked gate), explain the barrier clearly in the resubmission.

- Resubmit with Notes: Tag your update as a "Revision" and note exactly what was fixed.

Example from the Sources: If a dispute happens regarding occupancy, point back to your neutral indicators like "accumulated flyers" or "utilities present" to support your original finding.

Quick Win: Review the list of "Where disputes usually happen" (Address mismatch, missing side photos, unsupported occupancy) so you can double-check these items before hitting "Submit" on your first real job.

Warning Signs: Avoid getting defensive. If a coordinator asks for a redo, treat it as a chance to secure your spot on their "preferred" list by being the inspector who fixes things the fastest.

Analogy for Success: Think of these lessons like a pre-flight checklist for a pilot. You don't just jump in and fly; you check the fuel (tech prep), walk around the plane (four-side loop), and log the flight (upload proof). If you follow the checklist every time, you’ll have a smooth landing every time.