This response provides the three requested downloadable resources, formatted for easy, printable reference.

## 1. Pre-Inspection Equipment Checklist

### The Holy Trinity (Must-Haves)

| Item | Requirement/Standard | Source |
| --- | --- | --- |
| Primary Smartphone | High-quality camera. |  |
| Power Bank + Car Charger | Minimum 10,000mAh power bank and vehicle charger for dead batteries. |  |
| Measuring Tools | 25-foot minimum measuring tape, laser measure, and small ruler (for damage scale). |  |

### Secondary Essentials

| Item | Requirement/Standard | Source |
| --- | --- | --- |
| Flashlight/Headlamp | Essential for dark interiors (basements) and night exteriors. |  |
| Identification Badge | Must be visible on the outer side of your shirt/jacket/coat. |  |
| Notebook & Pen | Critical backup for field notes in case the app fails. |  |
| Clipboard | For managing pre-printed inspection forms and work orders. |  |
| Safety Vest | High-visibility vest for roadside or construction sites. |  |
| Dog Deterrent | Carry where lawful. |  |

### Optional Gear & Professional Presentation

| Item | Requirement/Standard | Source |
| --- | --- | --- |
| Weather Protection | Waterproof jacket/rain shell, insulated work gloves, wide-brim hat. |  |
| Footwear | Non-slip work boots with ankle support. |  |
| Access Keys | HUD keys or lockbox keys only when explicitly authorized. |  |
| Clothing | Business casual (khakis + polo). NO large logos. |  |

### Vehicle Emergency Supplies

| Item | Requirement/Standard | Source |
| --- | --- | --- |
| First Aid Kit | Must include bandages and antiseptic. |  |
| Emergency | Emergency flares or reflective triangles. |  |
| Maintenance | Jumper cables, tire pressure gauge. |  |
| Supplies | Emergency water, non-perishable snacks, trash bags. |  |

## 2. Photo Standards Quick Reference Card

### PHOTO ESSENTIALS: MINIMUM REQUIREMENTS

- Size: MINIMUM 1024x768 / Preferred 1600x1200.

- Orientation: ALL photos must be HORIZONTAL (Landscape).

### 💡 Lighting Rules

| Standard | Detail | Source |
| --- | --- | --- |
| Available Light | Use natural light first; open blinds if allowed. |  |
| Flash Use | Use flash in dark interiors (e.g., basements), but step back slightly to avoid "blown highlights." |  |
| Night Exteriors | Use a flashlight to fill in address numbers or meter faces for clarity. Take both a wide and a tight shot. |  |

### 📐 Framing Rules

| Standard | Detail | Source |
| --- | --- | --- |
| Context then Detail | Always capture a wide Context Shot (room, wall) first, followed by a tight Detail Shot (damage, meter). |  |
| Keep it Straight | Keep vertical lines vertical; avoid tilting. |  |
| Avoid Obstruction | Do not obstruct the image with fingers, straps, dashboard glare, or mirrors. |  |
| Address Centering | Center address numbers and include enough wall context to prove it is the subject property. |  |

### 💾 Metadata & Naming Rules

| Standard | Detail | Source |
| --- | --- | --- |
| Visible Stamps | NO People or time/date stamps visible on the photos themselves. |  |
| Metadata Check | Ensure your device time is set correctly and Location services (GPS validation) is enabled. |  |
| File Naming | Use a consistent pattern for quick sorting: ORDERID_ADDRESSCODE_SECTION_SEQUENCE_DESC.jpg (Example: 123456_742PINE_EXT_01_Front.jpg). |  |

### ⛔ Common Rejections & Privacy Violations

| Violation | Example | Source |
| --- | --- | --- |
| Through Glass/Mail | NEVER look or photograph inside any windows or mailboxes. |  |
| Blurry/Cropped | The address number is blurry or cropped (Bad Example), or the utility meter is unreadable (Bad Example). |  |
| Missing Context | Only a close-up of damage is provided without a wide wall shot showing context (Bad Example). |  |
| Photos of People | Document occupancy indicators (like cars or trash cans) without photographing people. |  |

## 3. Photo Rejection Troubleshooting Guide

Use this guide if a QC reviewer flags your photo as unacceptable.

| Problem | Cause/Bad Example | Solution (Right Path) | Source |
| --- | --- | --- | --- |
| Photo Too Dark/Underexposed | Image is dark, resulting in noise and unreadable details, especially in interiors (Bad Example) or under eaves (Bad Example). | Lighting Fix: Use your flashlight/headlamp to fill the area. Use the camera's flash, but step back slightly to prevent blown highlights. |  |
| Image is Blurry/Out of Focus | Subject (e.g., address number or meter face) is unreadable or blurry at 100% zoom. | Clarity Fix: Tap the screen to focus on the subject (e.g., the house numbers). Brace your phone/body against a steady object (wall/car) and retake 2-3 shots. Ensure the image is sharp at 100% zoom. |  |
| Missing Context for Damage | You only submitted a close-up of the damage (Bad Example). The QC reviewer cannot tell where the damage is located on the property. | Context Fix: Immediately take a wide shot of the entire wall or area (Context Shot). Follow up with the close-up, ideally using a ruler or tape measure for scale (Detail Shot). |  |
| No Time/GPS Data (Metadata Missing) | The portal cannot verify the location or time of the inspection. | Metadata Fix: Do NOT use an app to put a visible timestamp on the photo (prohibited). Instead, verify that device time is set correctly and Location services (GPS) are enabled in your device settings. |  |
| Vertical Photo Submitted | Any photo taken in portrait orientation is uploaded. | Format Fix: All photos MUST BE HORIZONTAL (landscape). Retake the photo immediately using the correct orientation. |  |
| Through Glass/Obstruction | You photographed the address through a shrub (Bad Example) or tried to photograph occupancy through a window (Bad Example). | Framing Fix: NEVER photograph or look inside windows. Remove obstructions (if possible) or change your angle until the required item (e.g., address, meter) is clearly centered and unobstructed. |  |
| Missing Mandatory Photo | You forgot one of the basic requirements (e.g., Close-Up of Roof, Electric Meter, Address). | Checklist Fix: Use the basic photo checklist (Address, Front, Back, Left, Right, Close-Up Roof, Electric Meter/Mast) before you leave the property. Retake the missing mandatory photo immediately. |  |