## Module Overview: Field Kit and Photo Standards

Target Audience: New inspectors ready to purchase equipment and take their first photos.

The goal of this module is simple: Set up your mobile office and camera settings to eliminate QC rejection and keep you safe. Think of this as helping you pack the ultimate contractor-grade loadout, built for speed, safety, and zero rework.

### 1. Learning Objectives

After completing this module, you will be able to:

- Assemble a complete, safe, and efficient field kit capable of handling various weather and property access scenarios.

- Configure your smartphone or camera to meet minimum resolution (1024x768) and preferred ratio (1600x1200 and 4:3) standards, and ensure all photos are horizontal.

- Implement proper lighting and framing techniques to capture both the context and the detail of required items and hazards.

- Capture readable and verifiable documentation for addresses, utility meters, and occupancy indicators.

- Establish consistent backup and file-naming procedures to protect your data if your app crashes or uploads fail.

### 2. Why This Matters (Preventing Rejections)

Proper equipment and adherence to photo standards directly translate to consistent evidence that survives quality control (QC) and disputes, which ultimately reduces rework.

- Safety and Professionalism: Having necessary gear like a high-visibility vest, non-slip boots, and professional clothing improves your safety and ensures you are representing the client well.

- Data Integrity: A good kit ensures you avoid rework caused by dead batteries, poor lighting, or missing measurement references. Using a consistent naming pattern (e.g., ORDERID_ADDRESSCODE_SECTION_SEQUENCE_DESC.jpg) helps prevent file mix-ups during submission.

- Verifiability: Your photos must be high-quality enough that someone else can identify the location and condition of the subject property without reading your notes. For instance, providing both a wide shot and a close-up of damage, potentially using a tape measure for scale, ensures documentation is complete and accurate.

### 3. Common Mistakes Beginners Make

#### Equipment & Preparation Mistakes:

- Underpowering the Day: Running out of battery because you didn't pack a portable power bank (minimum 10,000mAh recommended) or vehicle charger.

- Missing Context Tools: Forgetting essential measurement tools (tape measure, laser measure, ruler) needed for detailed documentation, especially for damage scale.

- Lacking Safety Gear: Not having basics like non-slip boots, a high-visibility vest, or dog deterrent (where lawful) for unpredictable environments.

- No Paper Backup: Relying only on the app. It's crucial to carry a small notepad and pen in case your app fails.

#### Photo & Technique Mistakes:

| Mistake Type | Bad Example | Solution/Standard |
| --- | --- | --- |
| Blur/Clarity | Taking photos where the address numbers are cropped or blurry. | Take a tight shot of the numbers and a wide shot for context, ensuring the image is sharp at 100% zoom. |
| Lighting (Interior) | Shooting dark interior views, resulting in noise or blur. | Use available light first (open blinds if allowed); use a flash in dark areas but step back slightly to avoid blown highlights. |
| Lighting (Exterior) | Capturing utility meters that are unreadable or blurry due to glare or darkness. | Use a flashlight to fill in address numbers at night (wide and tight shots); take a photo of the readable meter face plus a context shot. |
| Framing & Obstruction | Shooting from the driver’s seat with dashboard glare, or having fingers/straps obstruct the view. | Keep vertical lines vertical and avoid tilting; avoid obstructions like sun glare and mirrors. |
| Missing Context | Capturing only a close-up of damage without showing the context of the whole wall, or taking a photo of an occupied property that includes faces. | Always capture context then detail; provide a wide wall shot and then a close-up of damage with scale. Document occupancy indicators (like cars or trash cans) without photographing people. |
| Format | Submitting vertical photos. | ALL photos must be HORIZONTAL. |

### 4. Success Metrics

You have mastered this module when you can confidently answer "Yes" to these self-check questions for every submission:

- QC Check: Can someone else (like your FIM or QC reviewer) easily identify the exact location and condition of the item without reading your notes?

- Sharpness: Is the image sharp and in focus at 100 percent zoom?

- Completeness: Are all mandatory photos present (Address/House Numbers, Front, Back, Left, Right, Close-Up of Roof, Electric Meter, etc.)?

- Format Compliance: Are all photos horizontal (no vertical shots), and do they meet the minimum resolution standard?

- Metadata and Backup: Is your device time set correctly, location services enabled (if required), and have you followed your end-of-day backup strategy, keeping raw originals untouched?

### 5. Estimated Time to Complete

This information is not provided in the uploaded sources.

You need to treat your gear like a professional toolkit, not just a backpack. If your field kit is like a meticulously organized toolbox, you'll be able to fix the problem (capture the required data) the first time, preventing the "rework wrench" from being thrown into your schedule.